Firmware Upgrade Guide for Oscilloscope Series

The software version of the upgrade file must be greater than or equal to the current software version of the oscilloscope.

Warning: Do not power off the oscilloscope during the upgrade process.

Upgrade Steps

- Copy the corresponding upgrade file (e.g., mso3000hd_all_1.01.00XX_armv8a.ipk) that matches the oscilloscope model to a USB flash drive.

- Insert the USB flash drive into the USB port of the oscilloscope.

Upgrade Methods

Method 1

- Restart the oscilloscope and enter the upgrade interface.

- Use the multi-function rotary knob（or touchscreen） to select the upgrade file.

- Click on “Upgrades” to enter the upgrade process.

- When prompted to upgrade successfully, remove the USB flash drive and click “OK” on the screen, the oscilloscope will reboot.

- If prompted that the upgrade fails, please check whether the firmware corresponds to the current version.

- After reboot, check “Utility→About→Version” to make sure the upgrade is successful.

Method 2

- Navigate to the upgrade option in the Utility interface.

- Select the upgrade file that matches the oscilloscope model.

- Click the Upgrade button to start the process.

- Wait for the upgrade to be completed. If successful, the oscilloscope will automatically restart.

- Once finished, remove the USB flash drive.

- If prompted that the upgrade fails, please check whether the firmware corresponds to the current version.

- After reboot, check “Utility→About→Version” to make sure the upgrade is successful.